Press release dated August, 25th 2026

Versatile INDEX G160 and TRAUB TNX160 turn-mill centers

Ideal for small and complex components

The G160 turn-mill center, presented for the first time at the Open House 2026, sees INDEX round out the lower end of its current G range. At the AMB 2026 INDEX also presents the TRAUB variant TNX160. Conceptually, the machines are based on the previous G200.2 and offers a wide range of options in terms of equipment.

In recent years, INDEX has redesigned and re-engineered the turn-mill centers in the G-series and reorganized the model range. With the INDEX G160 and the TRAUB TNX160, the Esslingen-based machine manufacturer now rounds out the lower end of its turn-mill center range. These “small” models in the lineup are conceptually based on the previous G200.2, but can demonstrate significant improvements in milling. A special feature is the large selection of options in terms of spindles, tool carriers, and peripherals, enabling each customer to configure a machine tailored to their needs and optimized for cost.

Powerful main and counter spindles

The new INDEX G160 and TRAUB TNX160 turn-mill centers are equipped with a direct-drive, and therefore highly dynamic, main spindle, available in two sizes with bar clearances of 42 mm and 65 mm. Based on the technical specifications (speed 7,000/6,000 rpm, torque 65/170 Nm), the 42 mm version is suitable for smaller, high-speed components, while the larger version is also suitable for inserts that can be clamped in chucks up to 175 mm in diameter. On both of the machines, the counter spindle is identical in design to the main spindle, but features an additional Z axis.

Extensive tooling and tool carriers

The turn-mill centers can be equipped with two lower tool carriers, which are fitted as standard with a Y axis and can operate on both the main spindle and the counter spindle. The lower turrets feature 15 stations with a VDI 25 interface, all driven at speeds of up to 7,200 rpm (7.5 kW and 18.5 Nm at 25% DC). An interesting detail: Only the tools currently engaged in machining are supplied with cooling lubricant, contributing to high energy efficiency.

For the upper tool carrier, customers have access to a high-performance motor milling spindle with a B axis. The very compact, direct-drive milling spindle with HSK-T40 interface is available in two versions matched to the main spindles—with 32,000 rpm (17 Nm) and 18,000 rpm (29 Nm). Both versions allow internal coolant delivery of up to 80 bar, offering advantages for deep-hole drilling and difficult-to-machine materials.

The spindle is supplied from a newly developed tool magazine. This is a turret magazine with 60 or 90 tools. A double gripper automatically changes tools into the milling spindle.

Extensive automation options

The simplest form of automation is a bar loading magazine, for which the machines provides a UNIMAG 5 interface. This interface allows connection of the INDEX MBL 76 loader, available for 3 m or 4 m bars and offering excellent bar-guiding characteristics. In addition, the interface also supports third-party equipment. INDEX, for example, offers short bar loading magazines from Breuning.

For workpiece unloading, the classic solution is a linear unit that picks up the finished part and places it onto a conveyor belt. Significantly more options are provided by the integrated workpiece handling unit (WHU) developed by INDEX. It is equipped with three axes and can be used for loading and unloading as well as for remnant removal. High-end automation is available with the INDEX iXcenter, which supports fully automated operation with a pallet storage system. It is docked to the machine to then safely and flexibly load and unload blanks and/or finished parts. Downstream processes such as cleaning, measuring, and deburring can also be integrated in the robot cell.

For automated machining, it is essential to inspect and measure tools inside the machine. For this purpose, INDEX offers various systems, ranging from tactile methods to laser solutions that detect geometric changes and cutting-edge wear on the tools in the motor milling spindle without contact.

Additional peripheral options

INDEX also offers extensive options for peripheral equipment for the new machines. Several chip conveyors are available, including a KNOLL slat belt conveyor and a hinged-belt conveyor from Bürener. Both suppliers also provide various cooling lubricant solutions, which differ primarily in terms of filtration and high-pressure performance.

Both emulsion and oil can be used as cooling lubricants. In addition, INDEX offers suitable mist extractors: a mechanical separator for emulsion and an electrostatic system for oil.

The cooling concept of the INDEX G160 / TRAUB TNX160 includes two solutions: a 14 kW cooler that controls only the temperature of the machine, and a 20 kW cooling unit that additionally maintains the cooling lubricant at the desired temperature.

Another key focus is the energy efficiency of the machine. Here, INDEX takes a pioneering role through many detailed measures. These include energy regeneration during braking, the use of frequency-controlled pumps, counterbalancing of the vertical axes, and more (see box for further details).

Last but not least – and this is the biggest, or rather the only difference between the two new turn-mill centers - the INDEX G160 is equipped with the latest Siemens control system, Sinumerik ONE while the TRAUB relies on the proven Mitsubishi control system.

BOX

Key factors behind the high energy efficiency of INDEX machines

- High accelerations and rapid traverse rates reduce peripheral operating times (saving up to 50% of energy consumption)

- Energy recovery during braking through mains feedback

- Lower energy requirements due to intelligent material pairings and coatings with minimal friction

- Energy savings achieved through frequency-controlled pumps

- Reduced consumption through intelligent automatic shutdown of peripheral systems during machine idle time

- Spring-based counterbalancing in vertical feed axes reduces the energy demand of the servomotors

- Heat recovery through the provision of a central cooling lubricant interface

Photos: attached

Contact:	INDEX-Werke GmbH & Co. KG Hahn & Tessky

Rainer Gondek

Global Marketing Director

Phone: +49 (711) 3191-1286

rainer.gondek@index-werke.de